Team Members:
Ariella Morris (TL)

Prim Mangkhalathanakun

Stefanos Ioannidis

Adithya Mendis
Mathias Tidball

Eima Miyasaka

### Acronyms:

(For Section 7)

ANS — American Nuclear Society

D-LOFC — Depressurised Loss of Forced Cooling

DiD — Defence-in-Depth

HTGR — High Temperature Gas-cooled Reactor

HTTR — High Temperature Engineering Test Reactor

IAEA — International Atomic Energy Agency

INSAG — International Nuclear Safety Advisory Group

JAEA — Japan Atomic Energy Agency

LOFC — Loss of Forced Cooling

PyC — Pyrolytic Carbon

RCCS — Reactor Cavity Cooling System

RPV — Reactor Pressure Vessel

SCRAM — Safety Control Rod Axe Man (emergency reactor shutdown)

SiC — Silicon Carbide

TRISO — TRi-structural ISOtropic (coated fuel particle)

# Section 1 - Introduction and Project Overview

The global energy transition presents one of the most significant engineering challenges of our generation. Hard-to-abate sectors - aviation, shipping, and heavy freight - contribute approximately 30% of global energy-related emissions (Sharmina et al, 2020), yet remain largely inaccessible to direct electrification. Addressing this gap demands a fundamentally different approach: clean, energy-dense synthetic fuels produced at scale, without dependence on constrained resources.

This report presents a water-independent, nuclear-enabled Power-to-Liquids system in which a High Temperature Gas-cooled Reactor (HTGR) is coupled to a supercritical CO2 recompression Brayton cycle, direct air capture (DAC), high-temperature steam electrolysis (HTSE), and methanol synthesis.

The locked headline case used consistently in this final report is S2_30MW_FUELDISP under the fuel_displacement boundary. For this validated baseline, the model converges to a feasible plant at 30.0 MWth with 9.939 MWe net output, 33.90% thermal efficiency, 33.13% net efficiency, and 9,898 tCO2/year net reduction, corresponding to a primary figure of merit of 329.9 tCO2/MWth/year.

Earlier 36 MWth framing is retained only as historical development context and sensitivity evidence; it is not used as the headline final result basis.

Every design decision, from power conversion selection to carbon accounting methodology, was made through a documented, assumption-tracked process underpinned by rigorous modelling and independent verification and validation.

The report proceeds as follows: concept down-selection and architecture (Sections 2-3), modelling development and V&V (Sections 4-5), results and figures of merit (Section 6), safety philosophy (Section 7), and team structure, challenges and future work (Sections 8-9).

# Section 2 - Concept Overview and Down-Selection

### 2.1 Design Objective and Constraint

The design objective was to maximize defensible net CO2 reduction while preserving engineering feasibility under a strict water-independent heat rejection requirement. This made cooling architecture, process integration, and carbon accounting boundary definitions first-order design choices rather than secondary checks.

### 2.2 Application Selection (Pugh Matrix)

The application down-selection compared electricity export, hydrogen, and e-methanol pathways.

| Criterion | Weight | Electricity | H2 | E-methanol (selected) |
| --- | --- | --- | --- | --- |
| CO2 reduction potential | 5 | 0 | +1 | +2 |
| Water-independence support | 5 | 0 | -1 | +1 |
| Transportability/logistics | 4 | 0 | -2 | +2 |
| Heat-integration potential | 3 | 0 | +1 | +2 |
| Total |  | 0 | -1 | +7 |

Decision: e-methanol provided the strongest combined score across carbon impact, logistics practicality, and thermal-integration opportunity.

### 2.3 Carbon Capture and Power-Cycle Selections

Solid sorbent DAC was selected over high-temperature liquid-solvent capture due to direct compatibility with low-grade heat. Recompression sCO2 Brayton conversion was selected over steam Rankine due to dry cooling feasibility, compactness, and integration with the HTGR outlet temperature regime.

# Section 3 - Integrated System Architecture

The final architecture is split into three coupled zones: nuclear island, power conversion island, and process island.

- Nuclear island: HTGR thermal source with IHX as the physical isolation barrier.

- Power conversion island: recompression sCO2 cycle delivering net electrical power and rejected-heat streams.

- Process island: DAC, HTSE, and methanol synthesis under no-grid-export headline operation.

Thermal-cascade logic is retained: high-grade heat supports power conversion, lower-grade heat supports process duty before final rejection. This architecture converts what is typically treated as low-value rejected heat into a productive CO2-reduction resource.

# Section 4 - Modelling Development & Methodology

## 4.1 Modelling Philosophy and Fidelity

The modelling approach prioritizes thermodynamic consistency across subsystems rather than isolated subsystem optimization. Real-gas properties are evaluated with CoolProp to capture near-critical CO2 behavior where constant-property or ideal-gas assumptions are not reliable.

## 4.2 Coupled Solver and Feasibility Logic

The plant solve uses a coupled iterative structure with embedded feasibility gates:

- deterministic bounded operating-point seed search over (P_high, P_low, f_recomp),

- staged moderate tuning for infeasible starts,

- per-iteration diagnostics and convergence-reason tagging,

- fail-fast gating for headline infeasibility and canonical feasibility enforcement.

Feasibility constraints are checked during solve progression, not only as post-processing.

## 4.3 CO2 Accounting Methodology

The net annual CO2 calculation compares annual emissions in the baseline scenario to those in the project scenario, using explicit boundary labels to avoid ambiguity between accounting modes. To preserve continuity with the legacy team draft, the original baseline/project/net equation chain is retained in Appendix 4.3 as sensitivity/comparator context.

#### 4.3.1 Baseline Scenario

Two accounting boundaries are retained in the project, but only one is used for headline claims in the main body:

- fuel_displacement (headline): includes displaced fossil-fuel comparator assumptions.

- operational_only (sensitivity): net reduction from modeled plant operation and on-site process allocation.

#### 4.3.2 Project Scenario

Main-text equation (headline boundary):

Net_CO2_fuel_displacement = CO2_displaced_fossil + CO2_grid - CO2_embodied - (CO2_reemitted_synthetic if neutrality disabled)

For the locked headline case, grid export is zero by design and neutrality is enabled in the accounting settings, so:

Net_CO2_fuel_displacement = CO2_displaced_fossil - CO2_embodied = 9,898 tCO2/year

#### 4.3.3 Net CO2 Reduction

Sensitivity equation (reported in Section 6.3 only):

Net_CO2_operational_only = CO2_DAC + CO2_grid

This mode is retained for sensitivity context and is not used as the headline figure in the main report body.

#### 4.3.4 Key assumptions

- The main-body headline is always fuel_displacement.

- The operational_only basis is shown only as sensitivity.

- kgCO2 and kgCO2e are treated as equivalent in this engineering-stage accounting pass.

- No-grid-export policy is applied in the headline scenario.

# Section 5 - Verification and Validation

## 5.1 Verification

Verification focused on implementation correctness and control-flow integrity:

- deterministic operating-point search behavior and repeatability,

- feasibility-recovery range guards and metadata persistence,

- fail-fast and override pathway control logic,

- JSON-serializable diagnostics for trace artifacts.

These checks were executed through unit-level and integration-level tests in the branch workflow.

## 5.2 Validation Against Reference Case

The cycle reference validation remains the Dostal-style benchmark case. The committed authoritative run reports 41.07% thermal efficiency against a 45.0% +/- 5.0% acceptance band, and the validation gate passes.

## 5.3 Feasibility Evidence

The final authoritative run demonstrates:

- headline baseline (S2_30MW_FUELDISP) converged and feasible,

- plant energy closure within tolerance,

- all canonical scenarios S0-S3 converged and feasible under required gating.

This provides a branch-reproducible feasibility basis for the results in Section 6.

# Section 6 - Results & Figures of Merit

## 6.1 Plant Performance Summary

The headline performance case is the locked baseline S2_30MW_FUELDISP with fuel_displacement accounting. The integrated configuration is feasible and converged under the final authoritative run.

Table 6.1 — Integrated Plant Performance

| Metric | Value |
| --- | --- |
| Reactor thermal power | 30.0 MWth |
| Net electrical power | 9.939 MWe |
| Thermal efficiency | 33.90% |
| Net efficiency | 33.13% |
| Plant energy closure | 0.435% |
| CO2 mass flow | 165.9 kg/s |
| Compressor inlet temperature | 50.0 C |
| Turbine inlet temperature | 517.7 C |
| HTR pinch margin | +1.66 K |
| IHX pinch margin | +13.00 K |
| LTR pinch margin | +2.82 K |
| Helium return margin | +10.74 K |
| Feasible / Converged | True / True |

All electrical output remains on-site in headline mode, with no grid export.

## 6.2 Primary Figure of Merit

Table 6.2 – Primary Figure of Merit

| Metric | Value |
| --- | --- |
| Net CO2 reduction (fuel_displacement) | 9,898 tCO2/year |
| Primary FoM | 329.9 tCO2/MWth/year |
| DAC capture | 44,468 tCO2/year |
| Hydrogen production | 627.3 t/year |
| Methanol production | 3245.3 t/year |

The dominant sensitivity drivers include electrolyser efficiency (electricity demand per unit hydrogen), DAC parasitic load (energy required for CO2 capture), and overall cycle efficiency (heat-to-electric conversion performance). These parameters directly affect total energy demand and, therefore, the calculated CO2 reduction and FOM.

Uncertainty context from the committed canonical UQ artifact (120 samples): FoM mean 331.6 tCO2/MWth/year, p10/p50/p90 of 302.7 / 331.5 / 360.8, and a 95% interval of 288.6 to 377.9 tCO2/MWth/year.

## 6.3 Secondary Figures of Merit

Canonical sensitivity summary (all feasible and converged):

| Scenario | Basis | Net power (MWe) | FoM (tCO2/MWth/year) | Feasible |
| --- | --- | --- | --- | --- |
| S0_BASE_30MW_OPONLY | 30 MWth, operational_only | 9.939 | 1482.3 | Yes |
| S1_36MW_OPONLY | 36 MWth, operational_only | 12.101 | 1471.9 | Yes |
| S2_30MW_FUELDISP | 30 MWth, fuel_displacement (headline) | 9.939 | 329.9 | Yes |
| S3_36MW_FUELDISP | 36 MWth, fuel_displacement | 12.101 | 334.7 | Yes |

Secondary conclusions:

- Feasibility is maintained across all four canonical scenarios.

- 36 MWth results are retained as sensitivity context only.

- Operational-only values are sensitivity-only and not used as headline claims in the main report.

# Section 7 - Safety Philosophy

Safety is integral to our HTGR design. The architecture follows the IAEA Defence-in-Depth framework and relies on inherent reactor physics, robust fuel containment, and passive cooling. Quantitative transient values in Section 7.4 are explicitly provenance-labeled to distinguish branch-reproduced outputs from external-workstream evidence.

## 7.1 Defence-in-Depth Framework

The safety philosophy is structured around IAEA INSAG-10 Defence-in-Depth, comprising five independent levels: (1) Prevention through conservative design margins; (2) Detection and Control via instrumentation, negative temperature feedback, and control rod systems; (3) Engineered Safety Systems including TRISO fuel barriers and the passive RCCS; (4) Accident Management through RPV integrity and long-term passive heat removal; (5) Emergency Response which can be minimal for HTGRs, with reduced Emergency Planning Zones justified by the preceding layers’ effectiveness. Multiple barriers must fail simultaneously before any release occurs.

## 7.2 Inherent Safety Characteristics

Two physics-based mechanisms exist as the core of the safety case. A strongly negative temperature coefficient of reactivity (-3.5% mk·K-1) ensures that rising fuel temperatures suppress fission power without any operator action, via Doppler broadening of U-238 absorption cross-sections. The graphite core’s large thermal inertia (~55 tonnes, ~720 J·kg-1·K-1) causes transients to evolve over days. Both were experimentally confirmed during JAEA’s HTTR LOFC demonstrations.

## 7.3 TRISO Fuel and Passive Cooling

TRISO fuel particles provide four independent barriers to fission product release: porous carbon buffer, inner PyC, SiC, and outer PyC layers. The SiC coating retains fission products up to the 1,600°C design limit, verified by the German AVR, US AGR, and Chinese HTR-PM irradiation programmes. Decay heat is removed by a passive RCCS: natural-convection air cooling around the RPV requiring no pumps, valves, or AC power supply. This passive architecture satisfies the project’s water-independent heat rejection requirement.

## 7.4 Safety Transient Analyses

Three transient models were developed in Python to quantify safety margins under accident conditions. Each couples a lumped-parameter thermal model with point kinetics and the Wigner-Way decay heat approximation (7% of full power at shutdown, declining per ANS 5.1). Provenance labels are applied below for each quantitative claim.

### 7.4.1. Pressurised Loss of Forced Cooling (P-LOFC)

This bounding conservative case assumes helium remains in the primary circuit, no active SCRAM, and no natural convection. Peak fuel temperature reaches 1579 C at 38 hours (Figure 7.1), 21 C below the TRISO limit, before passively declining [External-workstream model result]. This is discussed in relation to HTTR LOFC literature and JAEA 2024 demonstration reports [Literature]. Omitting natural convection ensures the prediction is conservative.

[Figure 7.1. P-LOFC Transient: Peak Fuel Temperature vs. Time]

### 7.4.2 Depressurised Loss of Forced Cooling (D-LOFC)

The D-LOFC model adds primary circuit depressurisation, eliminating helium buoyancy-driven cooling. Heat removal relies solely on graphite conduction and radiation to the RCCS. Peak fuel temperature reaches 1590 C at 40 hours (Figure 7.2), confirming fuel integrity is maintained even under this more severe scenario [External-workstream model result].

[Figure 7.2. D-LOFC Transient: Peak Fuel Temperature vs. Time]

### 7.4.3 Natural Circulation Verification

To quantify the conservatism of Section 7.4.1, the P-LOFC model was re-run with natural circulation included (literature-based helium flow at ~3% of forced rate). Peak fuel temperature reduces to 1376 C (Figure 7.3), demonstrating 203 C of additional margin and confirming that the P-LOFC bounding case is appropriately conservative [External-workstream model result].

[Figure 7.3. P-LOFC with Natural Circulation: Temperature Comparison]

# Section 8: Challenges, Lessons Learned & Team Structure

## 8.1 Professional Challenges

The principal professional challenge was coordinating a genuinely multi-disciplinary team across reactor physics, thermodynamics, chemical engineering, and carbon accounting - each workstream operating under different modelling frameworks, timelines, and technical languages. Early in the project, it became clear that without a structured mechanism for cross-workstream alignment, assumptions made in one domain could silently invalidate work in another.

The response was twofold. First, a Master Assumptions Log was established as a single, version-controlled record of every major input, its source, its justification, and its sensitivity classification. Over 100 parameters were documented in this way, ensuring full traceability and reproducibility. A concrete example of this process in action was the boundary and headline lock to S2_30MW_FUELDISP with fuel_displacement accounting: once fixed, all FoM tables, canonical comparisons, and narrative claims were synchronized to a single source of truth. Second, a RACI framework was adopted to define clear ownership across the nuclear island, power cycle, DAC/HTSE, and CO₂ accounting workstreams - eliminating gaps and preventing duplication.

The key lesson was that rigorous documentation and communication infrastructure are not administrative overhead - they are engineering deliverables in their own right. In a systems-level project, the integration layer is where designs succeed or fail.

## 8.2 Technical Challenges

The most significant technical challenge was the non-linear behaviour of the sCO₂ Brayton cycle solver near the CO₂ critical point, where specific heat capacity varies by a factor of five across a narrow pressure-temperature window. Standard fixed-point iteration diverged under these conditions, and property discontinuities propagated errors across the coupled heat exchanger network.

The solution was a modular, staged validation approach: each component - turbine, compressors, recuperators, dry cooler - was verified in isolation against hand calculations before integration. The nested fixed-point iteration architecture was then built around this validated component library, with 11 embedded feasibility constraints checked at every iteration rather than post-hoc. Energy closure across the full cycle was confirmed to within 0.1%.

The lesson was that robust modelling is not a function of data quality alone - it is a function of structured simplification, transparent assumptions, and systematic uncertainty management. A model that converges confidently within defined bounds is more valuable than one that reaches a precise answer through opaque means.

## 8.3 Team Structure

The team comprised six members: Ariella Morris, Eima Miyasaka, Mathias Tidball, Prim Mangkhalathanakun, Stefanos Ioannidis, and Adithya Mendis. Responsibilities were divided by technical workstream with defined ownership: nuclear safety and transient modelling (Eima), power cycle modelling and V&V (Stef), CO₂ accounting and primary FOM (Prim), down-selection and system architecture (Mathias), results and secondary FOMs (Adithya), and report structure, team coordination, and Group Leader (Ariella). Weekly structured synchronisation meetings maintained alignment across workstreams, with the Master Assumptions Log serving as the shared technical ground truth. Responsibilities were distributed among all six members to reflect the genuine collaborative nature of the work.

# Section 9 - Novelty and Future Work

## 9.1 Novelty

The project novelty is not a single component, but the integrated use of heat quality across the full plant: power generation at high-grade heat, process duty at lower-grade heat, and explicit feasibility-gated operation under dry-cooling constraints. This design framing converts the dry-cooling penalty from a pure loss into a system-integration opportunity while keeping headline CO2 claims tied to an explicitly labeled comparator boundary (fuel_displacement).

## 9.2 Future Work

Priority next steps are:

- branch-reproducible safety transient coupling in the main plant repository,

- stabilization of fully coupled-cooler sensitivity mode at high ambient conditions,

- expanded uncertainty campaign consistent with full BEPU framing,

- higher-fidelity off-design heat-rejection analysis.

# References

(Section 1)

Sharmina, M., Edelenbosch, O. Y., Wilson, C., Freeman, R., Gernaat, D. E. H. J., Gilbert, P., … Le Quéré, C. (2021). Decarbonising the critical sectors of aviation, shipping, road freight and industry to limit warming to 1.5–2°C. Climate Policy, 21(4), 455–474. https://doi.org/10.1080/14693062.2020.1831430

(For Section 7)

[1] International Atomic Energy Agency (1996) Defence in Depth in Nuclear Safety. INSAG-10. Vienna: IAEA.

[2] Japan Atomic Energy Agency (2024) Success of Safety Demonstration Test in HTTR [Press release]. 28 March. Available at: https://www.jaea.go.jp/english/news/press/2024/032801/ (Accessed: 20 February 2026).

[3] Takamatsu, K., Tochio, D., Nakagawa, S. and Takada, S. (2014) 'Experiments and validation analyses of HTTR on loss of forced cooling under 30% reactor power', Journal of Nuclear Science and Technology, 51(11–12). doi:10.1080/00223131.2014.967324.

[4] Organisation for Economic Co-operation and Development/Nuclear Energy Agency (2024) Loss of Forced Coolant (LOFC) Project. Available at: https://www.oecd-nea.org/jcms/pl_25168 (Accessed: 24 February 2026).

[5] Idaho National Laboratory (no date) TRISO Fuel: Accident Performance and NRC Engagement. Report Sort_66160. Available at: https://inldigitallibrary.inl.gov/sites/sti/sti/Sort_66160.pdf (Accessed: 24 February 2026).

[6] International Atomic Energy Agency (2003) Evaluation of High Temperature Gas Cooled Reactor Performance. TECDOC-1382. Vienna: IAEA.

[7] American Nuclear Society (2014) Decay Heat Power in Light Water Reactors. ANS 5.1-2014. La Grange Park, IL: ANS.

[8] Generation IV International Forum (2018) VHTR Safety Assessment. GIF/VHTR. Available at: https://www.gen-4.org/gif/upload/docs/application/pdf/2018-12/gifvhtr_safety_assessment_finaldec2018.pdf (Accessed: 24 February 2026).

(For Sections 4.3, 6.2, and Appendix 4.3)

EUROCONTROL. (n.d.). 7 Amount of emissions released by fuel burn. EUROCONTROL. Retrieved https://ansperformance.eu/economics/cba/standard-inputs/latest/chapters/amount_of_emissions_released_by_fuel_burn.html

ICAP. (2026, January 19). EU Emissions Trading System (EU ETS) | International Carbon Action Partnership. https://icapcarbonaction.com/en/ets/eu-emissions-trading-system-eu-ets

Le Boulch, D., Buronfosse, M., Le Guern, Y., Duvernois, P.-A., & Payen, N. (2024). Meta-analysis of the greenhouse gases emissions of nuclear electricity generation: Learnings for process-based LCA. The International Journal of Life Cycle Assessment, 29(5), 857–872. https://doi.org/10.1007/s11367-024-02293-y

MissionZero. (n.d.). Sustainable CO2: What it is, where it comes from, what it means for the climate. Retrieved 19 January 2026, from https://www.missionzero.tech/lab-notes/sustainable-co2

Rolls-Royce Corporation. (2021, October). FAA Continuous Lower Energy, Emissions, and Noise (CLEEN II) Technologies Program. https://www.faa.gov/sites/faa.gov/files/2022-02/phase2_rolls-royce_sustainable_aviation_fuels_final.pdf

Scott, D. (2025, October 6). Why SAF is expected to play a larger role in near- and medium-term decarbonization than zero-emission aircraft. International Council on Clean Transportation. https://theicct.org/why-saf-is-expected-to-have-a-larger-role-in-near-and-medium-term-decarbonization-than-zero-emission-aircraft-sept25/

# Appendix

## Appendix 4.3: Detailed CO2 Calculation and Derivations

Sensitivity-only note: this appendix is provided for legacy equation continuity and comparator context. In this final report, fuel_displacement is the headline basis and operational_only is a sensitivity basis.

Symbol mapping to current report boundary labels:

- Legacy E_Baseline and E_Project terms map to comparator accounting in fuel_displacement headline mode.

- Main-body sensitivity uses Net_CO2_operational_only = CO2_DAC + CO2_grid.

- Legacy net result (~33,374 tCO2/year) is historical comparator context and not the current solved headline value.

Table 4.3: Input parameters and numerical substitutions

#### Derivation of Nuclear Heat EF

The nuclear heat emission factor is derived from the nuclear electricity emission factor and the HTGR efficiency of our model (Liu et al., 2020).